TRƯỜNG ĐẠI HỌC DUY TÂN

KHOA CÔNG NGHỆ THÔNG TIN

BÁO CÁO THỰC HÀNH

ĐỀ TÀI: Chia địa chỉ IPv4

Sinh viên thực hiện: Nguyễn Văn Duy Mạnh

Mã sinh viên           : 29211100024

Lớp                          : CMU -  CS 252 DIS

Giảng viên               : Ths Lê Văn Tịnh

# BÀI TẬP THƯC HÀNH

## Câu 1: Cho hệ thống mạng gồm 228 Host và địa chỉ IP được thiết lập ở lớp 192.168.1.0/24.

Hãy chia hệ thống mạng này thành bốn mạng con (Net 1: có 120 Host, Net 2: có 60 Host, Net 3: có 30 Host và Net 4: có 18 Host) gồm các thông tin: Network ID (địa chỉ lớp mạng con), Subnet Mask (mặt nạ của mạng con), Start IP Address (địa chỉ IP bắt đầu của mạng con), End IP Address(địa chỉ IP kết thúc mạng con), Broadcast IP(địa chỉ IP quảng bá của mạng con).

Giải:

Để chia hệ thống mạng 192.168.1.0/24 thành bốn mạng con với số lượng host như yêu cầu, ta cần tìm các thông số sau cho mỗi mạng con: Network ID, Subnet Mask, Start IP Address, End IP Address và Broadcast IP.

Đầu tiên, ta sẽ tính số lượng bit subnet cần cho từng mạng con:

1. Net 1: 120 Host

Số lượng bit subnet cần: log2(120 + 2) ≈ 7 bits

2. Net 2: 60 Host

Số lượng bit subnet cần: log2(60 + 2) ≈ 6 bits

3. Net 3: 30 Host

Số lượng bit subnet cần: log2(30 + 2) ≈ 5 bits

4. Net 4: 18 Host

Số lượng bit subnet cần: log2(18 + 2) ≈ 5 bits

Sau đó, ta sẽ chia địa chỉ IP gốc thành các mạng con dựa trên số lượng bit subnet đã tính:

### Bước 1: Chia Network 192.168.1.0/24 thành mạng con:

1. Net 1: 192.168.1.0/25 (2^7 = 128, với 128 IP, có thể đủ cho 120 Host)

2. Net 2: 192.168.1.128/26 (2^6 = 64, với 64 IP, có thể đủ cho 60 Host)

3. Net 3: 192.168.1.192/27 (2^5 = 32, với 32 IP, có thể đủ cho 30 Host)

4. Net 4: 192.168.1.224/27 (Với số lượng IP còn lại sau khi chia cho các mạng trên, sẽ cần 5 bits nữa để đủ cho 18 Host)

### Bước 2: Xác định các thông số cho từng mạng con:

1. Net 1:

- Network ID: 192.168.1.0

- Subnet Mask: 255.255.255.128 (/25)

- Start IP Address: 192.168.1.1

- End IP Address: 192.168.1.126

- Broadcast IP: 192.168.1.127

2. Net 2:

- Network ID: 192.168.1.128

- Subnet Mask: 255.255.255.192 (/26)

- Start IP Address: 192.168.1.129

- End IP Address: 192.168.1.190

- Broadcast IP: 192.168.1.191

3. Net 3:

- Network ID: 192.168.1.192

- Subnet Mask: 255.255.255.224 (/27)

- Start IP Address: 192.168.1.193

- End IP Address: 192.168.1.222

- Broadcast IP: 192.168.1.223

4. Net 4:

- Network ID: 192.168.1.224

- Subnet Mask: 255.255.255.224 (/27)

- Start IP Address: 192.168.1.225

- End IP Address: 192.168.1.254

- Broadcast IP: 192.168.1.255

## Câu 2: Người ta ghi nhận địa chỉ IP của một Host như sau 113.160.111.143/19. Anh (chị) hãy cho biết:

- Host trên thuộc mạng có chia mạng con không? Nếu có thì bao nhiêu mạng con và bao nhiêu host trên mỗi mạng

- Hãy cho biết địa chỉ đường mạng chứa host

- Hãy cho biết địa chỉ Broadcast của mạng đó và liệt kê danh sách host

Giải:

a. Để xác định xem có chia mạng con hay không, ta cần tính số lượng mạng con và số lượng host trên mỗi mạng. Với subnet mask /19, ta có thể tính như sau:

- Số lượng bit host: 32 (tổng số bit) - 19 (bit mạng) = 13 bit

- Số lượng host trên mỗi mạng: 2^(số bit host) - 2 (do có 2 địa chỉ mạng và broadcast không sử dụng được) = 2^13 - 2 = 8192 - 2 = 8190 host

Vậy, ta có 1 mạng với 8190 host.

b. Địa chỉ đường mạng chứa host có thể được tính bằng cách đặt tất cả các bit host thành 0. Vì vậy, địa chỉ đường mạng là: 113.160.96.0

c. Địa chỉ broadcast của mạng này là địa chỉ mà tất cả các bit host đều là 1. Vì vậy, địa chỉ broadcast là: 113.160.127.255

Danh sách host trên mạng này bắt đầu từ 113.160.96.1 đến 113.160.127.254.

## Câu 2: Một hệ thống mạng có khoảng 1000 nút mạng, được cấp phát nguồn địa chỉ IP trong đó có host có địa chỉ 150.150.41.20

Hệ thống đó cần chia 13 mạng con. Anh (chị) hãy tiến hành cấp phát địa chỉ IP cho mạng trên và cho biết địa chỉ đường mạng của host, địa chỉ broadcast của mạng và liệt kê danh sách host của mạng đó.

Giải:

Để chia mạng thành 13 mạng con, chúng ta cần sử dụng một mặt nạ mạng phù hợp. Với 13 mạng con, chúng ta cần ít nhất 4 bit (2^4 = 16) để đánh số các mạng con. Do đó, chúng ta có thể sử dụng mặt nạ mạng /28 (255.255.255.240) để chia mạng.

Với mặt nạ mạng /28, mỗi mạng con sẽ có 16 địa chỉ IP (2^4), trong đó 14 địa chỉ có thể được sử dụng cho các host (trừ địa chỉ mạng và địa chỉ broadcast).

Danh sách các mạng con khi chia mạng 150.150.41.0/24 với mặt nạ /28:

- Mạng con 1:

  - Địa chỉ mạng: 150.150.41.0

  - Địa chỉ broadcast: 150.150.41.15

  - Danh sách host: từ 150.150.41.1 đến 150.150.41.14

- Mạng con 2:

  - Địa chỉ mạng: 150.150.41.16

  - Địa chỉ broadcast: 150.150.41.31

  - Danh sách host: từ 150.150.41.17 đến 150.150.41.30

- Mạng con 3:

  - Địa chỉ mạng: 150.150.41.32

  - Địa chỉ broadcast: 150.150.41.47

  - Danh sách host: từ 150.150.41.33 đến 150.150.41.46

- Mạng con 4:

  - Địa chỉ mạng: 150.150.41.48

  - Địa chỉ broadcast: 150.150.41.63

  - Danh sách host: từ 150.150.41.49 đến 150.150.41.62

- Mạng con 5:

  - Địa chỉ mạng: 150.150.41.64

  - Địa chỉ broadcast: 150.150.41.79

  - Danh sách host: từ 150.150.41.65 đến 150.150.41.78

- Mạng con 6:

  - Địa chỉ mạng: 150.150.41.80

  - Địa chỉ broadcast: 150.150.41.95

  - Danh sách host: từ 150.150.41.81 đến 150.150.41.94

- Mạng con 7:

  - Địa chỉ mạng: 150.150.41.96

  - Địa chỉ broadcast: 150.150.41.111

  - Danh sách host: từ 150.150.41.97 đến 150.150.41.110

- Mạng con 8:

  - Địa chỉ mạng: 150.150.41.112

  - Địa chỉ broadcast: 150.150.41.127

  - Danh sách host: từ 150.150.41.113 đến 150.150.41.126

- Mạng con 9:

  - Địa chỉ mạng: 150.150.41.128

  - Địa chỉ broadcast: 150.150.41.143

  - Danh sách host: từ 150.150.41.129 đến 150.150.41.142

- Mạng con 10:

  - Địa chỉ mạng: 150.150.41.144

  - Địa chỉ broadcast: 150.150.41.159

  - Danh sách host: từ 150.150.41.145 đến 150.150.41.158

- Mạng con 11:

  - Địa chỉ mạng: 150.150.41.160

  - Địa chỉ broadcast: 150.150.41.175

  - Danh sách host: từ 150.150.41.161 đến 150.150.41.174

- Mạng con 12:

  - Địa chỉ mạng: 150.150.41.176

  - Địa chỉ broadcast: 150.150.41.191

  - Danh sách host: từ 150.150.41.177 đến 150.150.41.190

- Mạng con 13:

  - Địa chỉ mạng: 150.150.41.192

  - Địa chỉ broadcast: 150.150.41.207

  - Danh sách host: từ 150.150.41.193 đến 150.150.41.206

## Câu 3: Cho địa chỉ IP của một số Host như sau:

Trong những host trên host nào nằm cùng mạng con. Hãy chỉ cho biết địa chỉ mạng con chứa host đó, địa chỉ Broadcast của mạng và liệt kê danh sách host hợp lệ.

Cho địa chỉ IP 100.200.100.200/19. Hãy cho biết:

- Host trên thuộc mạng có chia mạng con không? Nếu có thì bao nhiêu mạng con, bao nhiêu host?

- Hãy cho biết địa chỉ đường mạng chứa host

- Hãy cho biết địa chỉ Broadcast và liệt kê danh sách host hợp lệ

Giải:

Đầu tiên, chúng ta sẽ xác định địa chỉ mạng con từ địa chỉ IP và subnet mask:

Địa chỉ IP: 100.200.100.200 Subnet mask: /19

Subnet mask 19 bit có dạng: 11111111.11111111.11100000.00000000

Địa chỉ mạng con: 100.200.96.0

Địa chỉ broadcast: 100.200.127.255

Bước tiếp theo là kiểm tra xem địa chỉ IP của host (100.200.100.200) có nằm trong phạm vi mạng con không.

100.200.100.200 - 100.200.96.0 = 4

100.200.127.255 - 100.200.96.0 = 31

Vì 4 nằm giữa 0 và 31 nên host thuộc mạng con.

Sau khi xác định được host nằm trong mạng con, chúng ta sẽ tính toán các thông tin còn lại:

- Địa chỉ mạng: 100.200.96.0

- Địa chỉ broadcast: 100.200.127.255

- Danh sách host hợp lệ:

  - Từ 100.200.96.1 đến 100.200.127.254

Đối với câu hỏi về việc chia mạng con và số lượng host, chúng ta có thể tính toán như sau:

Số lượng bit mạng (subnet bits): 19 Số lượng bit host: 32 - 19 = 13

Số lượng mạng con: 2^số bit mạng = 2^19 = 524288 mạng Số lượng host trên mỗi mạng con: 2^số bit host - 2 = 2^13 - 2 = 8190 host

Vậy đáp án cho câu hỏi là:

- Host thuộc mạng con.

- Có 524288 mạng con.

- Mỗi mạng con có 8190 host.

- Địa chỉ mạng chứa host là 100.200.96.0.

- Địa chỉ broadcast của mạng là 100.200.127.255.

- Danh sách host hợp lệ: từ 100.200.96.1 đến 100.200.127.254.

## Câu 4: Giả sử ta có một địa chỉ Host là: 172.16.40.32/255.255.240.0

- Host trên nằm trong mạng có chia mạng con hay không? Nếu có thì có bao nhiêu địa chỉ mạng con và bao nhiêu địa chỉ Host trong mạng con này?

- Tìm địa chỉ mạng của địa chỉ host trên?

- Tìm địa chỉ Broadcast địa chỉ host trên?

- Tìm dãy địa chỉ Host từ địa chỉ mạng con tìm được ở câu b?

Giải:

a. Để xác định xem có chia mạng con hay không, chúng ta cần xem xét phần mạng con của subnet mask. Nếu có ít nhất một bit trong phần mạng con được đặt là 0, tức là mạng này đã được chia thành các mạng con.

Trong subnet mask 255.255.240.0, phần mạng con có 12 bit là 1 (tức là /20), và phần dành cho host có 20 bit là 0. Do đó, mạng này đã được chia thành các mạng con.

Số bit cho mạng con: 20 Số bit cho host: 32 - 20 = 12

Số mạng con có thể tạo ra: 2^12 = 4096 Số địa chỉ host trong mỗi mạng con: 2^(32-20) - 2 = 4094 (trừ 2 địa chỉ mạng và broadcast)

b. Địa chỉ mạng của địa chỉ host trên: 172.16.32.0

c. Địa chỉ broadcast của địa chỉ host trên:172.16.47.255

d. Dãy địa chỉ host trong mạng con được tìm thấy ở câu (b) là từ 172.16.32.1 đến 172.16.47.254.

## Câu 5: Cho địa chỉ IP: 192.168.10.210/27.

- Địa chỉ trên thuộc lớp nào? Giải thích?

- Tìm địa chỉ mạng và địa chỉ broadcast của mạng chứa địa chỉ IP trên?

- Hãy chia mạng con vừa tìm được ở câu b thành 4 mạng con. Liệt kê các địa chỉ mạng, địa chỉ broadcast và dãy địa chỉ host của 4 mạng con này.

Giải:

a. Địa chỉ IP 192.168.10.210/27 thuộc lớp C. Lớp mạng được xác định bằng cách kiểm tra phần đầu của địa chỉ IP. Trong trường hợp này, phần đầu là 192, điều này phù hợp với lớp C của địa chỉ IP, với dải mạng 192.0.0.0 - 223.255.255.255

b. Để tìm địa chỉ mạng và địa chỉ broadcast, ta cần biết rằng /27 nghĩa là 27 bit đầu tiên của địa chỉ IP được sử dụng cho địa chỉ mạng. Vì vậy, địa chỉ mạng sẽ là 192.168.10.192 và địa chỉ broadcast sẽ là 192.168.10.223.

c. Để chia mạng con, ta cần thêm 2 bit vào phần mạng. Điều này tạo ra 4 mạng con như sau:

- Mạng con thứ nhất: Địa chỉ mạng 192.168.10.192/29, địa chỉ broadcast 192.168.10.199, dãy địa chỉ host từ 192.168.10.193 đến 192.168.10.198.

- Mạng con thứ hai: Địa chỉ mạng 192.168.10.200/29, địa chỉ broadcast 192.168.10.207, dãy địa chỉ host từ 192.168.10.201 đến 192.168.10.206.

- Mạng con thứ ba: Địa chỉ mạng 192.168.10.208/29, địa chỉ broadcast 192.168.10.215, dãy địa chỉ host từ 192.168.10.209 đến 192.168.10.214.

- Mạng con thứ tư: Địa chỉ mạng 192.168.10.216/29, địa chỉ broadcast 192.168.10.223, dãy địa chỉ host từ 192.168.10.217 đến 192.168.10.222.

## Câu 6: Một công ty sử dụng địa chỉ mạng là 192.168.5.0 với Subnet Mask là 255.255.255.0 cho hệ thống máy tính trong công ty. Công ty có 5 Phòng ban, mỗi Phòng ban có 20 máy tính. Ban lãnh đạo công ty muốn mỗi phòng ban là 1 mạng riêng dựa trên địa chỉ mạng của công ty.

- Hãy tiến hành chia mạng con dựa vào địa chỉ mạng ở trên và đáp ứng các yêu cầu trên của công ty.

- Xác định Subnet Mask của các mạng con.

- Mỗi mạng con có bao nhiêu IP có thể sử dụng.

- Liệt kê địa chỉ đầu và địa chỉ cuối của các mạng con.

Giải:

a. Để chia mạng con, ta cần thêm 3 bit vào phần mạng (vì 2^3 = 8 > 5, số lượng mạng con cần tạo). Điều này tạo ra 8 mạng con như sau:

- Mạng con thứ nhất: Địa chỉ mạng 192.168.5.0/27

- Mạng con thứ hai: Địa chỉ mạng 192.168.5.32/27

- Mạng con thứ ba: Địa chỉ mạng 192.168.5.64/27

- Mạng con thứ tư: Địa chỉ mạng 192.168.5.96/27

- Mạng con thứ năm: Địa chỉ mạng 192.168.5.128/27

b. Subnet Mask của các mạng con sẽ là 255.255.255.224 (vì 27 bit đầu tiên được sử dụng cho địa chỉ mạng).

c. Mỗi mạng con có 32 IP, nhưng chỉ có 30 IP có thể sử dụng (vì địa chỉ mạng và địa chỉ broadcast không được sử dụng để gán cho các thiết bị trong mạng).

d. Địa chỉ đầu và địa chỉ cuối của các mạng con:

- Mạng con thứ nhất: Địa chỉ đầu 192.168.5.1, địa chỉ cuối 192.168.5.30

- Mạng con thứ hai: Địa chỉ đầu 192.168.5.33, địa chỉ cuối 192.168.5.62

- Mạng con thứ ba: Địa chỉ đầu 192.168.5.65, địa chỉ cuối 192.168.5.94

- Mạng con thứ tư: Địa chỉ đầu 192.168.5.97, địa chỉ cuối 192.168.5.126

- Mạng con thứ năm: Địa chỉ đầu 192.168.5.129, địa chỉ cuối 192.168.5.158

TRẮC NGHIỆM

2^n = số mạng con với n là số bit netID

2^m -2 = số host usable với m là số bit hostID

- – Assume that 4 bits have been borrowed. Identify the subnet addresses (choose 3)

a 192.168.14.8

b 192.168.14.16

c 192.168.14.24

d 192.168.14.32

e 192.168.14.148

f 192.168.14.208

- – Assuming a subnet mask of 255.255.224.0, which of the following would be a valid host address? (choose 3)

a 124.78.103.0

b 125.67.32.0

c 125.78.160.0

d 126.78.48.0

e 176.55.96.0

f 186.211.100.0

- – Which of the following are private IP addresses? ( choose 3)

a 172.168.33.1

b 10.35.66.70

c 192.168.99.5

d 172.18.88.90

e 192.169.77.89

f 127.33.55.16

- – The router reads each bit to determine the class of an address. Which of the following binary numbers would the router identify as a public class A address? (choose 3)

a 00001010.10101100.11001100.00000111

b 00011111.11110011.11111111.00111011

c 01011101.11100001.11001100.11011011

d 10000000.11111000.11000111.11110011

e 00010111.11011011.11000001.11001100

- – What is the maximum number of subnets that can be assigned to networks when using the address

172.16.0.0 with a subnet mask of 255.255.240.0

a 16

b 32

c 30

d 14

e this is an invalid subnet mask for the Network

- – Which network mask should you place on a class C address to accommodate a user requirement of two sub networks with a maximum of 35 hosts on each network?

A 255.255.255.192

B 255.255.255.224

C 255.255.255.240

D 255.255.255.248

network mask = subnet mask

- – How many valid host IP addresses are available on the following network/subnetwork? 198.197.196.16/3

a 2

b 30

c 254

d 16,382

e 65,534

8- Given an IP address of 172.16.2.160 and a subnet mask of 255.255.255.192, to which subnet does the host belong?

A 172.16.2.32

B 172.16.2.64

C 172.16.2.96

D 172.16.2.128

E 172.16.2.192

- – Given the following IP address from the class B address range:

172.35.21.12

Your network plan requires no more than 126 hosts on a subnet that includes this address. When

you configure the IP address in Cisco IOS software, which value should you use as the subnet mask?

A 255.255.0.0

B 255.255.128.0

C 255.255.255.128

D 255.255.255.252

10- You are given an ip adress 132.15.136.2/18 what subnet is the host ip on?

A 132.15.136.0

B 132.15.128.0

C 132.15.192.0

D 132.15.64.0

11- In order to have 5 subnets on each subnet, how many bits of subnetting will you use on the class B address 162.13.0.0/16?

A 255.255.128.0

B 255.255.224.0

C 255.255.240.0

D 255.255.248.0

Subnet Exercises

Note: You need to thoroughly understand the structure of Class A’s, B’s, and C’s network address in order to handle the following questions. Good lucks. I do not have answer keys to these questions. You are welcome to ask me to check your answers.

- Given the subnet mask, 255.255.255.0, and an IP address 66.68.100.1,

  - How many bits are used for subnet ID? Why?

  - How many IP addresses are lost due to the subnetting?

Giải:

a. The subnet mask 255.255.255.0 is a /24 subnet mask. This means that 24 bits are used for the subnet ID. The reason is that each octet in the subnet mask that is filled with all 1s (255 in decimal) represents 8 bits of the subnet ID. So, three octets of 255 equal 24 bits (3*8=24).

b. When subnetting, two IP addresses are typically lost per subnet. One is used as the network address and the other as the broadcast address. The network address represents the subnet itself, and the broadcast address is used to send data to all hosts on the subnet. Therefore, in this case, 2 IP addresses are lost due to subnetting.

- In dealing with a network configuration problem. An IT manager found that the following information on a station: IP address was 225.101.10.97 and the subnet mask was 255.255.255.224. What might be the problem?

Giải:

The problem lies in the IP address. The IP address `225.101.10.97` is not a valid IPv4 address. The reason is that the first octet of an IPv4 address must be in the range of `0-223`. The number `225` is outside of this range.

In the context of IPv4, each IP address consists of four octets (numbers between 0 and 255) separated by periods (also known as 'dots'). Each octet represents 8 bits of the address, and they add up to a 32-bit address space.

The subnet mask `255.255.255.224` is valid. It represents a `/27` subnet in CIDR notation, which means it allows for `30` usable hosts in the network.

So, the IT manager should assign a valid IP address to the station to resolve the network configuration problem. For example, an IP address like `223.101.10.97` would be valid and could be used.

- In dealing with a network configuration problem. An IT manager found that the following information on a station: IP address was 199.101.10.32 and the subnet mask was 255.255.255.192. Assume the network ID is correct. What might be the problem?

Giải:

The IP address `199.101.10.32` and the subnet mask `255.255.255.192` are both valid. The subnet mask `255.255.255.192` corresponds to a `/26` subnet in CIDR notation, which means it allows for `62` usable hosts in the network.

However, the problem might be that the IP address `199.101.10.32` is a network address. In an IP network, the network address is the first IP address of the network and is used for routing. It cannot be assigned to a host (a device such as a computer or printer). The last address of the network is the broadcast address, which is used to communicate with all hosts on the network. Neither the network nor the broadcast address can be assigned to a host.

In this case, `199.101.10.32` is the network address for the subnet `199.101.10.32/26`. The valid host addresses for this subnet are from `199.101.10.33` to `199.101.10.62`, and the broadcast address is `199.101.10.63`.

So, the IT manager should assign a valid host address (not the network or broadcast address) to the station to resolve the network configuration problem. For example, an IP address like `199.101.10.33` would be valid and could be used.

- For a particular Class C subnet, if the subnet ID is 0100, answer the following questions:

  - Define the subnet mask.

  - Does 212.34.23.129 belong to this subnet?

  - How many subnets can the organization have?

  - How many IP addresses are available in each segment?

  - How many IP addresses are lost due to subnetting (in terms of the whole organizations)?

Giải:

a.  For a Class C subnet, if the subnet ID is 0100, it means we are using 4 bits for subnetting. The subnet mask would be `255.255.255.240` This is because the binary for 240 is `11110000`, where the subnet bit is 1, and the host bit is 0

b. The IP address `212.34.23.129` does not belong to this subnet. This is because the first octet of the IP address is 212, which falls in the range of Class C addresses (192.0.0.0 - 223.255.255.255). However, the subnet we are considering is for a specific Class C network, not all Class C networks.

c. If we are using 4 bits for subnetting in a Class C network, the number of subnets would be `2^4 = 16`

d. Each subnet would have `2^(8-4) - 2 = 14` usable IP addresses. The `-2` is because the first address is used as the network address and the last address is used as the broadcast address

e. In a Class C network, there are originally `2^8 = 256` addresses After subnetting with 4 bits, we have 16 subnets each with 14 usable addresses, so we have `16*14 = 224` usable addresses. Therefore, the number of addresses lost due to subnetting is `256 - 224 = 32`. These lost addresses are used for network and broadcast addresses in each subnet.

- For a particular Class B subnet, if the subnet ID is 001000:

a. How many bits are used for subnet ID?

b. Define the subnet mask.

c. Does 144.33.48.254 belong to this subnet?

d. How many subnets can the organization have?

e. How many IP addresses are available in each segment?

f. How many IP addresses are lost due to subnetting (in terms of the whole organizations)?

Giải:

a. The subnet ID you provided is 001000, which is 6 bits long.

b. For a Class B network, the default subnet mask is 255.255.0.0. If we’re using 6 bits for subnetting, the subnet mask would be 255.255.252.0 in decimal form.

c. To determine if an IP address belongs to a subnet, we need to know the full subnet address, not just the subnet ID. With only the subnet ID 001000, we can’t definitively say whether 144.33.48.254 belongs to this subnet.

d. With 6 bits used for subnetting, the organization can have 2^6 = 64 subnets.

e. In a Class B network, there are 16 bits available for host addresses. If 6 bits are used for subnetting, that leaves 10 bits for host addresses. So, each subnet can have 2^10 = 1024 addresses. However, we must subtract 2 addresses for the network address and broadcast address, leaving 1024 - 2 = 1022 usable addresses per subnet.

f. The total number of addresses in a Class B network is 2^16 = 65,536. If we’re using 6 bits for subnetting, we have 64 subnets each with 1024 addresses, for a total of 64 * 1024 = 65,536 addresses. So, no addresses are “lost” due to subnetting. However, each subnet has 2 addresses (network and broadcast) that can’t be assigned to hosts, so 64 * 2 = 128 addresses are unusable for host assignment. Therefore, you could say that 128 addresses are “lost” due to subnetting.

- Divide the IP network address, 195.25.2.0, so that it is capable of supporting at least 9 network segments.

Giải:

To divide the IP network address 195.25.2.0 into at least 9 network segments, we need to borrow enough bits from the host portion of the IP address to accommodate the required number of networks.

1. Subnet mask: 255.255.255.240 (/28)

2. Number of bits borrowed: 4

3. Number of subnets: 2^4 = 16

4. Number of usable addresses per subnet: 2^(32-28) - 2 = 14

Now, let's allocate the subnets:

1. Subnet 1: 195.25.2.0/28

2. Subnet 2: 195.25.2.16/28

3. Subnet 3: 195.25.2.32/28

4. Subnet 4: 195.25.2.48/28

5. Subnet 5: 195.25.2.64/28

6. Subnet 6: 195.25.2.80/28

7. Subnet 7: 195.25.2.96/28

8. Subnet 8: 195.25.2.112/28

9. Subnet 9: 195.25.2.128/28

- Divide the IP network address, 140.25.0.0, so that it is capable of supporting 254

Giải:

To divide the IP network address 140.25.0.0 so that it is capable of supporting 254 hosts per subnet, we need to calculate the appropriate subnet mask that allows for at least 254 usable IP addresses in each subnet.

Given that we need at least 254 hosts per subnet, we can calculate the number of bits required for the host portion of the IP address. Since 2^8 = 256, and we need at least 254 hosts, we need at least 8 bits for the host portion.

Now, let's calculate:

1. Number of bits required for hosts: 8

2. Number of bits remaining for the network portion: 32 - 8 = 24

So, the subnet mask we need will be a /24 subnet mask, which is 255.255.255.0.

This gives us 256 subnets (from 140.25.0.0 to 140.25.255.0), each with 254 usable host addresses. The IP ranges for the first three subnets would be:

Subnet 1: 140.25.0.0 to 140.25.0.255

Subnet 2: 140.25.1.0 to 140.25.1.255

Subnet 3: 140.25.2.0 to 140.25.2.255

And so on, up to Subnet 255: 140.25.255.0 to 140.25.255.255.

- Divide the IP network address, 140.25.0.0, so that it is capable of supporting 128 network segments.

Giải:

The IP address 140.25.0.0 is a Class B address, which has a default subnet mask of 255.255.0.0. This allows for one network with 65,534 hosts.

To create 128 subnets, we need to determine the appropriate subnet mask. The number of subnets created is given by 2^n, where n is the number of bits borrowed from the host portion of the address.

To get 128 subnets, we need to borrow 7 bits (since 2^7 = 128). This leaves us with 9 bits for hosts, giving us 2^9 - 2 = 510 hosts per subnet (we subtract 2 for the network address and broadcast address).

So, the new subnet mask would be 255.255.254.0 or /23 in CIDR notation (since we have 23 bits for the network and subnet, and 9 bits for hosts).

This gives us 128 subnets, each with 510 usable host addresses. The IP ranges for the first three subnets would be:

Subnet 1: 140.25.0.0 to 140.25.1.255

Subnet 2: 140.25.2.0 to 140.25.3.255

Subnet 3: 140.25.4.0 to 140.25.5.255

And so on, up to Subnet 128: 140.25.254.0 to 140.25.255.255.

- Divide IP network address, 25.25.0.0, so that it is capable of supporting 65,354 network segments.

Giải:

To divide the IP network address 25.25.0.0 into segments capable of supporting 65,354 network segments, we need to determine the appropriate subnet mask.

To support 65,354 network segments, we need to borrow all 16 bits from the host portion.

The default subnet mask for a Class A network (like 25.25.0.0) is 8 bits for the network portion (the first 8 bits) and 24 bits for the host portion (the last 24 bits). We need to change this to accommodate the additional 16 bits for subnets.

So, the new subnet mask will be 8 (original network bits) + 16 (additional bits) = 24 bits for the network portion.

Thus, the new subnet mask will be 255.255.255.0.

This means that the first 24 bits of the IP address will be for the network portion, and the remaining 8 bits (32 - 24) will be for the host portion.

Now, we have 8 bits for hosts, which gives us \(2^8 - 2 = 254\) usable hosts per subnet (minus 2 for the network address and the broadcast address).

However, since we're using all 16 bits for subnetting, each subnet will contain only two IP addresses (the network address and the broadcast address).

So, the resulting network segments will be:

- Subnet 1: 25.25.0.0/32

- Subnet 2: 25.25.0.2/32

- Subnet 3: 25.25.0.4/32

- ...

- Subnet 65,354: 25.25.255.252/32

Each of these subnets will contain only two IP addresses (network address and broadcast address), and there will be 65,354 such subnets.

10. Create subnets for IP network address, 222.22.22.0, so that one segment can support at least 56 attachments and 5 segments at least 12 attachments each. You also need to define the subnet mask for each segment.

Giải:

For the segment with at least 56 attachments: To get at least 56 hosts, we need 6 bits for host addresses (since 2^6 - 2 = 62 which is more than 56). This means we are using 26 bits for the network and subnet (since IP addresses are 32 bits long), leaving 6 bits for hosts. So, the subnet mask for this segment would be 255.255.255.192 or /26 in CIDR notation.

For the 5 segments with at least 12 attachments each: To get at least 12 hosts, we need 4 bits for host addresses (since 2^4 - 2 = 14 which is more than 12). This means we are using 28 bits for the network and subnet, leaving 4 bits for hosts. So, the subnet mask for these segments would be 255.255.255.240 or /28 in CIDR notation.

11. Create subnets for IP network address, 200.10.10.0, so that one segment can support at least 48 attachments and 3 segments at least 30 attachments each. You also need to define the subnet mask for each segment.

Giải:

For the segment with at least 48 attachments: To get at least 48 hosts, we need 6 bits for host addresses (since 2^6 - 2 = 62 which is more than 48). This means we are using 26 bits for the network and subnet (since IP addresses are 32 bits long), leaving 6 bits for hosts. So, the subnet mask for this segment would be 255.255.255.192 or /26 in CIDR notation.

For the 3 segments with at least 30 attachments each: To get at least 30 hosts, we need 5 bits for host addresses (since 2^5 - 2 = 30). This means we are using 27 bits for the network and subnet, leaving 5 bits for hosts. So, the subnet mask for these segments would be 255.255.255.224 or /27 in CIDR notation.